Table of Contents

# 📊 GB Power Market Data Inventory - Complete Summary

Generated: 6 November 2025
Project: inner-cinema-476211-u9
Dataset: uk_energy_prod

## 🎯 Executive Summary

You have 49 tables and 2 views containing comprehensive GB electricity market data spanning 2022 to 2025. This includes: - Market prices & imbalances (155K+ records) - Generation fuel mix (5.7M+ records, 20 fuel types) - System frequency data - Balancing mechanism data - Bid-offer data - Demand forecasts - Interconnector flows

## 📈 Currently Active Data Sources

### 1. bmrs_mid - Market Index Data (System Prices) ⭐ CURRENTLY USED

What it contains: System Sell Price (SSP) and System Buy Price (SBP)

| Metric | Value |
| --- | --- |
| Total Records | 155,405 rows |
| Date Range | 2022-01-01 to 2025-10-30 |
| Days of Data | 1,375 days (~3.8 years) |
| Price Range | £-102.92 to £1,561.59/MWh |
| Average Price | £53.55/MWh |
| Total Volume | 145.2 million MWh |
| Granularity | Per settlement period (30-min intervals) |
| Update Frequency | Daily (T+2 settlement lag) |

Key Fields: - settlementDate - Trading date - settlementPeriod - Period 1-48 (or 1-50 on clock change) - price - Market price (£/MWh) - volume - Traded volume (MWh) - dataset - Data source identifier

Use Cases: - ✅ Current: Daily arbitrage analysis - Price volatility tracking - Market pattern identification - Revenue optimization

### 2. bmrs_fuelinst - Instantaneous Fuel Generation Mix ⭐ READY TO USE

What it contains: Real-time generation by fuel type across GB

| Metric | Value |
| --- | --- |
| Total Records | 5,691,925 rows |
| Date Range | 2022-12-31 to 2025-10-30 |
| Days of Data | 1,035 days (~2.8 years) |
| Fuel Types | 20 different types |
| Granularity | 5-minute intervals |

Fuel Types Available (all 20): 1. CCGT - Combined Cycle Gas Turbines (297K records) 2. WIND - Wind generation (297K records) 3. NUCLEAR - Nuclear power (297K records) 4. BIOMASS - Biomass generation (297K records) 5. COAL - Coal generation (297K records) 6. NPSHYD - Non-pumped storage hydro (297K records) 7. PS - Pumped storage (297K records) 8. OCGT - Open Cycle Gas Turbines (297K records) 9. OIL - Oil generation (297K records) 10. OTHER - Other sources (297K records)

Interconnectors (imports/exports): 11. INTFR - France interconnector (297K records) 12. INTIRL - Ireland interconnector (297K records) 13. INTNED - Netherlands interconnector (297K records) 14. INTEW - East-West interconnector (297K records) 15. INTIFA2 - IFA2 interconnector (297K records) 16. INTNEM - Nemo interconnector (297K records) 17. INTELEC - ElecLink interconnector (297K records) 18. INTNSL - North Sea Link (297K records) 19. INTVKL - Viking Link (204K records, started Jul 2023) 20. INTGRNL - Greenlink (130K records, started Mar 2024)

Key Fields: - settlementDate - Trading date - fuelType - Type of generation/import - generation - MW output - startTime - Timestamp

Use Cases: - Renewable penetration analysis - Carbon intensity calculation - Import/export tracking - Generation mix trends - Correlation with prices

### 3. bmrs_freq - System Frequency Data

What it contains: GB electricity system frequency measurements

| Metric | Value |
| --- | --- |
| Total Records | 0 (empty table) ⚠️ |
| Status | Not currently populated |

Key Fields: - measurementTime - Timestamp - frequency - System frequency (Hz)

Note: This table exists but is currently empty. Would need to be populated from BMRS API if required.

### 4. bmrs_bod - Bid-Offer Data

What it contains: Balancing mechanism bid and offer prices

| Metric | Value |
| --- | --- |
| Status | Available but not yet analyzed |
| Contains | Unit-level bid/offer pairs |

Key Fields: - timeFrom, timeTo - Time range - bmUnit - Balancing mechanism unit - bidPrice, offerPrice - Prices (£/MWh) - bidVolume, offerVolume - Volumes (MW)

Use Cases: - Generator behavior analysis - Balancing cost forecasting - Unit-level profitability

## 🗂️ Complete Table Inventory (49 Tables)

### Balancing Mechanism (5 tables)

- balancing_acceptances - Accepted balancing actions

- balancing_dynamic_sel - Dynamic selection

- balancing_nonbm_volumes - Non-BM balancing volumes

- balancing_physical_mels - Maximum Export Limits

- balancing_physical_mils - Maximum Import Limits

### BMRS Core Data (20+ tables)

- bmrs_mid ⭐ - Market Index Data (ACTIVE)

- bmrs_fuelinst ⭐ - Fuel generation mix (5.7M rows)

- bmrs_bod - Bid-Offer Data

- bmrs_freq - System frequency (empty)

- bmrs_boalf - Bid-Offer Acceptance Level Flagged

- bmrs_costs - System costs

- bmrs_demand_forecast - Demand forecasts

- bmrs_disbsad - Disaggregated Balancing Services Adjustment Data

- bmrs_fou2t14d - Forecasts 2-14 days ahead

- bmrs_fou2t3yw - Forecasts 2 weeks-3 years ahead

- bmrs_fuelhh - Half-hourly fuel data

- bmrs_imbalngc - Imbalance prices NGC

- bmrs_inddem - Indicated demand

- bmrs_indgen - Indicated generation

- bmrs_indo - Indicated imbalance

- bmrs_itsdo - Interconnector data

- bmrs_mdp - Market depth prices

- bmrs_mdv - Market depth volumes

- bmrs_melngc - MEL NGC

- bmrs_mnzt - Minute zero time

- bmrs_mzt - MZT data

- bmrs_ndf - Net demand forecast

- bmrs_ndfd - Net demand forecast daily

- bmrs_ndfw - Net demand forecast weekly

- bmrs_ndz - Net demand zero

- bmrs_netbsad - Net BSAD

- bmrs_nonbm - Non-BM data

- bmrs_nou2t14d - National output usable forecasts

- bmrs_nou2t3yw - National output usable forecasts long-term

- bmrs_ntb - Notice to deliver bids/offers

### IRIS Alternative Data (10+ tables)

Alternative/additional data sources with _iris suffix: - bmrs_beb_iris - BEB IRIS data - bmrs_boalf_iris - BOALF from IRIS - bmrs_bod_iris - Bid-Offer from IRIS - bmrs_freq_iris - Frequency from IRIS - bmrs_fuelinst_iris - Fuel generation from IRIS - bmrs_inddem_iris - Indicated demand from IRIS - bmrs_indgen_iris - Indicated generation from IRIS - bmrs_indo_iris - Indicated imbalance from IRIS - bmrs_mels_iris - MEL from IRIS - bmrs_mid_iris - Market Index from IRIS - bmrs_mils_iris - MIL from IRIS

### Unified Views (2 views)

- bmrs_boalf_unified - Combined BOALF data

- bmrs_fuelinst_unified - Combined fuel generation

### Deduplicated Tables (1 table)

- bmrs_fuelinst_dedup - Deduplicated fuel generation data

### Bid-Offer Data (1 table)

- bid_offer_data - Historical bid-offer information

## 🔍 Data Granularity & Time Periods

### Settlement Period Structure

- Normal Days: 48 settlement periods (00:00-23:30)

- Each Period: 30 minutes

- Clock Change Days: 46 periods (spring) or 50 periods (autumn)

### Data Frequencies

| Data Type | Update Frequency | Granularity |
| --- | --- | --- |
| Market Prices (MID) | Daily (T+2) | 30-minute (per SP) |
| Fuel Generation | Every 5 minutes | 5-minute snapshots |
| System Frequency | Real-time | Second-by-second (when populated) |
| Bid-Offer Data | Per settlement | 30-minute |
| Demand Forecasts | Daily | Various (hourly, daily, weekly) |

## 📊 Data Quality Summary

### bmrs_mid (Market Prices)

- ✅ Completeness: 1,375 days of continuous data

- ✅ Consistency: No duplicates

- ✅ Accuracy: Validated against BMRS source

- ⚠️ Timeliness: ~7 day lag due to settlement process

- ✅ Coverage: All settlement periods captured

### bmrs_fuelinst (Generation Mix)

- ✅ Completeness: 1,035 days of data

- ✅ Coverage: All 20 fuel types

- ✅ Frequency: 5-minute resolution

- ✅ Volume: 5.7 million records

- ⚠️ Note: Viking Link (INTVKL) starts Jul 2023, Greenlink (INTGRNL) starts Mar 2024

### Known Data Gaps

- bmrs_freq: Empty table (needs population)

- Settlement Lag: All BMRS data has T+2 publishing delay

- Historical Gap: Some tables start 2022, others 2023

- Interconnector History: Newer links have shorter history

## 💡 Data Enhancement Opportunities

### 1. Immediate Use - Already Available

✅ Add Generation Mix to Daily Analysis

# Example: Correlate wind generation with prices
query = """
SELECT 
  DATE(m.settlementDate) as date,
  m.settlementPeriod as sp,
  AVG(m.price) as avg_price,
  SUM(CASE WHEN f.fuelType = 'WIND' THEN f.generation ELSE 0 END) as wind_mw,
  SUM(CASE WHEN f.fuelType = 'SOLAR' THEN f.generation ELSE 0 END) as solar_mw,
  SUM(CASE WHEN f.fuelType = 'CCGT' THEN f.generation ELSE 0 END) as gas_mw
FROM `inner-cinema-476211-u9.uk_energy_prod.bmrs_mid` m
LEFT JOIN `inner-cinema-476211-u9.uk_energy_prod.bmrs_fuelinst` f
  ON DATE(m.settlementDate) = DATE(f.settlementDate)
WHERE DATE(m.settlementDate) >= DATE_SUB(CURRENT_DATE(), INTERVAL 14 DAY)
GROUP BY date, sp
"""

### 2. Carbon Intensity Calculation

Use fuel mix data to calculate: - Real-time carbon intensity (gCO2/kWh) - Renewable penetration % - Zero-carbon generation %

### 3. Price Forecasting

Combine data sources: - Historical prices (bmrs_mid) - Weather patterns (wind/solar generation) - Demand forecasts (bmrs_demand_forecast) - Interconnector flows (INT* fuel types)

### 4. Arbitrage Refinement

- Identify low-price periods correlated with high wind

- Predict negative prices based on renewable generation

- Factor in generation mix for storage decisions

## 🎯 Recommended Next Steps

### Priority 1: Enhance Current Analysis (Low Effort, High Value)

- Add generation mix to daily battery_arbitrage.py

  - Wind/solar correlation with negative prices

  - Gas generation as price indicator

  - Carbon intensity tracking

### Priority 2: Historical Analysis (Medium Effort, High Value)

- Backtest 3+ years of data

  - Identify seasonal patterns

  - Model price-generation relationships

  - Calculate optimal arbitrage strategies

### Priority 3: Forecasting (High Effort, High Value)

- Build predictive models

  - Next-day price forecasting

  - Renewable generation impact

  - Interconnector flow effects

### Priority 4: Real-time Monitoring (Medium Effort, Medium Value)

- Populate bmrs_freq table

  - System frequency monitoring

  - Grid stability indicators

  - Balancing event detection

## 📋 Data Schema Quick Reference

### bmrs_mid (System Prices)

dataset          STRING    -- Data source identifier
startTime        DATETIME  -- Window start
settlementDate   DATETIME  -- Trading date
settlementPeriod INTEGER   -- Period 1-48/50
price            FLOAT     -- Market price £/MWh
volume           FLOAT     -- Volume MWh

### bmrs_fuelinst (Generation Mix)

settlementDate   DATETIME  -- Trading date
fuelType         STRING    -- CCGT, WIND, NUCLEAR, etc.
generation       FLOAT     -- MW output
startTime        DATETIME  -- Measurement time

### bmrs_bod (Bid-Offer Data)

timeFrom         DATETIME  -- Period start
timeTo           DATETIME  -- Period end
bmUnit           STRING    -- Balancing unit
bidPrice         FLOAT     -- Bid price £/MWh
offerPrice       FLOAT     -- Offer price £/MWh
bidVolume        FLOAT     -- Bid volume MW
offerVolume      FLOAT     -- Offer volume MW

## 💰 Storage Costs

### Current BigQuery Storage

- Total Dataset Size: ~50-100 MB (estimated)

- Storage Cost: $0.00/month (within 10 GB free tier)

- Query Cost: ~$0.0009/month (current usage)

- Total Cost: < $0.01/month 🎉

### If You Add More Data

- Next 10 GB: Still free

- After 20 GB: $0.02/GB/month

- Query costs: $5/TB processed (first 1TB free)

## 🔗 Data Relationships

bmrs_mid (Prices)
    │
    ├── Same settlementDate/Period ──┐
    │                                 │
bmrs_fuelinst (Generation)           │
    │                                 │
    └── Correlation Analysis ────────┘
                │
                ├── High Wind → Low/Negative Prices
                ├── High Demand → High Prices
                └── Low Gas → Variable Prices

bmrs_bod (Bids/Offers)
    │
    └── Unit-level detail for prices

bmrs_demand_forecast
    │
    └── Forward-looking price drivers

## 📞 Data Access Commands

### List All Tables

bq ls --format=pretty inner-cinema-476211-u9:uk_energy_prod

### Check Table Schema

bq show --schema inner-cinema-476211-u9:uk_energy_prod.TABLE_NAME

### Sample Data

bq query --use_legacy_sql=false "
  SELECT * FROM \`inner-cinema-476211-u9.uk_energy_prod.TABLE_NAME\`
  LIMIT 10
"

### Export to CSV

bq extract \
  --destination_format=CSV \
  inner-cinema-476211-u9:uk_energy_prod.TABLE_NAME \
  gs://your-bucket/output.csv

## 📚 Documentation References

- MASTER_SYSTEM_DOCUMENTATION.md - Complete system architecture

- BIGQUERY_COMPLETE.md - BigQuery setup details

- BIGQUERY_OPTIMIZATION_ANALYSIS.md - Query optimization

- battery_arbitrage.py - Current analysis script

Summary: You have a comprehensive 3.8-year dataset covering prices, generation, and market operations. The bmrs_mid table is actively used daily. The bmrs_fuelinst table (5.7M rows, 20 fuel types) is ready to add immediate value by correlating generation mix with prices. Total cost remains under 1 cent/month. 🚀

Last Updated: 2025-11-06